Driver installation Guide for NetWare v3.12 server

========================================================

Introduction

============

This document describes the procedure to install ODI LAN drivers on the

Novell Netware 3.12 Server for AEF-380TXD PCI Fast Ethernet Adapter.

Installation Procedure

======================

A. Patch your server 3.12 is the first step :

Because this version of driver follows Novell ODI Driver Specification

Version 3.3, the old server must be patched to work well with this

driver. The patch method is described as below:

1. Copy LOADER.EXE and LSWAP.EXE to the NetWare 3.12 server directory,

the directory must be \NWSERVER (if not, rename the directory name).

2. Execute LSWAP.EXE at \NWSERVER.

B. Installing driver procedures for Novell NetWare v3.12 server :

1. Copy MSM31X.NLM, ETHERTSM.NLM and DC21X4.LAN from \netware\3.12

directory of the driver diskette to the hard disk directory where

NetWare 3.12 system files are locatted.

2. Boot NetWare file server by executing "SERVER" as you usually do.

3. To load the driver manually, simply at the system console screen

type the following command:

: Load NBI31X.NLM

: Load MSM31X.NLM

: Load ETHERTSM.NLM

: Load DC21X4 Frame=Ethernet_xxxxx LINE_SPEED=xxx

or, you can directly load file from the driver disk :

: Load A:\NETWARE\3.12\NBI31X.NLM

: Load A:\NETWARE\3.12\MSM31X.NLM

: Load A:\NETWARE\3.12\ETHERTSM.NLM

: Load A:\NETWARE\3.12\DC21X4 Frame=Ethernet_xxxxx LINE_SPEED=xxx

Note: (1) NBI31X.NLM, MSM31X.NLM, ETHERTSM.NLM can be auto-loaded

when loading LAN driver.

(2) Frame type depend on your network frame type.

(Default frame type is Ethernet_802.2).

(3) LINE_SPEED value is 10 or 100 only for DC21140 Based Card.

10 : 10MBps, 100 : 100MBps   (Default speed is 100 Mbps)

4. To bind IPX to the adapter driver. Type:

: Bind IPX to DC21X4 Net=xxxxx

Note:  Net value depend on your network configuration.

5. To load the driver automatically when server start-up, you must

create or edit AUTOEXEC.NCF.

--- Sample AUTOEXEC.NCF file ---

(1) One PCI board with multiple frame :

REM Load the MSM and TSM on Netware 3.12

REM ------------------------------------

load nbi31x

load msm31x

load ethertsm

REM Working with DC21X4 driver (one PCI board)

REM ------------------------------------------

load DC21X4 frame=ethernet_802.2 name=e1

load DC21X4 frame=ethernet_802.3 name=e2

load DC21X4 frame=ethernet_ii    name=e3

load DC21X4 frame=ethernet_snap  name=e4

bind ipx to e1 net=10

bind ipx to e2 net=20

bind ipx to e3 net=30

bind ipx to e4 net=40

(2) Mutiple PCI board with multiple frame :

REM Load the MSM and TSM on Netware 3.12

REM ------------------------------------

load nbi31x

load msm31x

load ethertsm

REM Working with DC21X4 driver (multiple PCI boards )

REM -------------------------------------------------

load DC21X4 frame=ethernet_802.2 name=e1   slot=16

load DC21X4 frame=ethernet_802.3 name=e2   slot=16

load DC21X4 frame=ethernet_ii    name=e3   slot=16

load DC21X4 frame=ethernet_snap  name=e4   slot=16

bind ipx to e1 net=10

bind ipx to e2 net=20

bind ipx to e3 net=30

bind ipx to e4 net=40

load DC21X4 frame=ethernet_802.2 name=e5   slot=17

load DC21X4 frame=ethernet_802.3 name=e6   slot=17

load DC21X4 frame=ethernet_ii    name=e7   slot=17

load DC21X4 frame=ethernet_snap  name=e8   slot=17

bind ipx to e5 net=50

bind ipx to e6 net=60

bind ipx to e7 net=70

bind ipx to e8 net=80

Parameters

==========

The following parameters belong on the server's command line, or in

the load line inside the AUTOEXEC.NCF. The general way of using these

parameters is:

LOAD <driver name> <PARAMETER NAME>... <PARAMETER NAME[=VALUE]>

Example:

LOAD DC21X4 FRAME=Ethernet_802.2 LINE_SPEED=100

Driver specific parameters list  (as for NetWare standard parameters,

please see the NetWare user's manual):

---------------------------------------------------------------------

AUI 	       selects AUI port.

TP		       selects TP port.

BNC 	       selects BNC port.

AUTODETECT	       enables autodetection of media type when driver

is loading. Note that this ONLY! works at

initialization time, i.e. when the driver first

loads for a network card based on DC21041.

TP_NO_LINK_TEST    selects twisted pair connection, but disables

the LINK test for specific HUBs that do not

support LINK.

TP_FULL_DUPLEX     selects twisted pair connection to work in

full duplex mode.

TURBO	       Fixes the problem of multiple transmit

underflows and receive overflows (i.e. lose of

packets) in systems that use certain PCI

chipsets.

BIOS16	       instructs the driver to access the PCI BIOS

using 16-bit real mode calls, instead of the

PCI32 mechanism.

This is not recommended, and should be used

ONLY! on motherboards that have a known problem

in the PCI BIOS implementation.

LINE_SPEED	       selects line speed between 10MPBs and 100MBPs

on network cards based on DC21140.

Format: LINE_SPEED=<value>

Allowed values are: 10,100

TX_THRESHOLD       Range 0..4.

0 =  72 bytes DEF = 0

1 =  96 bytes

2 = 128 bytes

3 = 160 bytes

4 = 160 bytes, store and forward mode

Format: TX_THRESHOLD=<value>

(If using this card for 486 PC, set this parameter

as "TX_THERSHOLD=4")