Installation Guide for Remote Boot-ROM (RPL)

=================================================

Introduction

============

This document describes the procedure to use the Remote Boot-ROM

for AEF-380TXD PCI Fast Ethernet Adapter.

Installation Procedures

=======================

1. Make a bootable diskette, place that diskette into floppy driver A.

2. Copy the required files to that diskette, for example, "HIMEM.SYS",

"EMM386.EXE", "COMMAND.COM" ....

3. Copy "LSL.COM", "DC21X4.COM", "IPXODI.COM", "NETX.EXE" (or "VLM.EXE"

and all VLM files) to that diskette.

4. Create "AUTOEXEC.BAT", "CONFIG.SYS" and "NET.CFG" files.

5. Login NetWare as supervisor.

6. Execute following commands :

MAP G:=SYS:SYSTEM <Enter>

MAP F:=SYS:LOGIN <Enter>

7. Change to the F drive, then Execute following command :

G:DOSGEN <Enter>

8. If your DOS version is 5.0 or higher, execute following command :

RPLFIX NET$DOS.SYS <Enter>

9. Copy "AUTOEXEC.BAT" file from the boot diskette to the LOGIN directory.

10. At NetWare Server console, execute following commands:

(Supposing server use DC21X4 PCI adapter)

: load DC21X4 frame=Ethernet_802.2

: bind IPX to DC21X4

: load RPL

: bind RPL to DC21X4   (Choosing Ethernet_802.2 frame type)

11. Last, restart workstation using remote BootROM function to connect

NetWare Server.

--- Sample CONFIG.SYS file ---

:

DEVICE=SETVER.EXE

LASTDRIVE=Z      --------> only for VLM

:

--- Sample AUTOEXEC.BAT file ---

:

LSL

DC21X4           ---------> ODI driver

IPXODI

NETX             ---------> or VLM

F:LOGIN

:

--- Sample NET.CFG file ---

:

LINK DRIVER DC21X4

FRAME ETHERNET_802.2

FRAME ETHERNET_802.3

NETWARE DOS REQUESTER

FIRST NETWORK DRIVE = F