Driver installation Guide for NetWare DOS ODI client

==========================================================

Introduction

============

This document describes the procedure to install Netware DOS ODI

driver for AEF-380TXD PCI Fast Ethernet Adapter.

Installation Procedure

======================

1. Copy the following files to a user specified directory:

copy a:\NETWARE\DOSODI\*.*

2. If using VLM, also copy the following files:

copy a:\NETWARE\DOSODI\VLM\*.*

3. Edit the NET.CFG file if necessary to select protocol and

media type options.

4. To connect to a server, use the following loading sequence:

LSL

DC21X4

IPXODI

NETX    or VLM

LOGIN

Keywords of NET.CFG

===================

Optional Bus type and Slot Number Keywords:

-------------------------------------------

BUS ID   PCI 4       Forces the driver to search for the board

on PCI bus only.

SLOT     1           Forces the driver to search for the board

instance. (1 is First Board).

Optional DC21X4 driver keywords:

--------------------------------

TURBO                Disable PCI post write buffer in Intel PCI chipset

which gives the PCI bus higher priority over the

other busses (EISA or ISA).

TP                   Selects Twisted Pair interface.

TP_FULL_DUPLEX       Selects Twisted Pair interface running in FullDuplex

mode.

AUTOSENSE            Performs Power-Up Autosense on all types of boards

in order to connect to the active line,

and dynamic 10/100 autosensing (at run-time) on boards

Supporting it.

RX_BUFFERS   5       Allows the user to select the amount of receive buffers

to work with (between 2 to 16).

More buffers means larger driver resident in memory but

with higher performance.

TX_THRESHOLD 4       Range 0..4.

0 =  72 bytes DEF = 0

1 =  96 bytes

2 = 128 bytes

3 = 160 bytes

4 = 160 bytes, store and forward mode

(If using this card for 486 PC, set this parameter

as "TX_THERSHOLD=4")

optional keywords for 10 MBps devices

-------------------------------------

AUI                  Selects AUI interface.

BNC                  Selects BNC interface.

TP_No_Link_Test      Selects Twisted Pair interface. Transmit does not

perform link pulses test.

T4                   Selects 100 base T4 interface

(in conjunction with Line_Speed  100)

optional keywords for 100 MBps devices

--------------------------------------

Line_Speed  100      Selects the LAN speed 10 or 100 (10MBps or 100MBps).

Default is 100MBps for the Fast Ethernet.

SNOOZE               Changes Chip's Power management to Power-Saving mode.

TX_THRESHOLD       Range 0..4.

0 =  72 bytes DEF = 0

1 =  96 bytes

2 = 128 bytes

3 = 160 bytes

4 = 160 bytes, store and forward mode

Format: TX_THRESHOLD=<value>

(If you are using this card for 486 PC, set this

parameter as "TX_THERSHOLD=4")