Driver Installation Guide for MS LAN Manager v2.x (OS2)

=============================================================

Introduction

============

This document describes the procedure to setup NDIS 2.01 driver on

LAN Manager 2.x OS/2 station for AEF-380TXD PCI Fast Ethernet Adapter.

This driver conforms to the Network Driver Interface Specification

(NDIS 2.01) defined by Microsoft/3Com.

Installation Procedure

======================

A. Suppose you install MS LAN Manager on a OS2 workstation for the

first time.

1. Execute the "SETUP.EXE" program that is provided by LAN Manager.

Upon completion of retrieval of the last workstation disk's

contents, you will be asked if you want to import additional

network drivers.  Answer "YES".

2. Insert this diskette into the floppy drive.

3. Follow the instructions on the screen. "SETUP.EXE" program will

retrieve NDIS driver programs from the diskette and put them on

the directory where LAN Manager resides.

4. Reboot your computer and follow NDIS binding procedure to use

the DC21X4.OS2 NDIS driver programs.

B. If you have installed LAN Manager in your harddisk.

1. Execute the "SETUP.EXE" program.

2. Insert this Driver diskette into the floppy drive.

3. On the screen, select 'Configuration', 'Network Drivers', 'other

Drivers' function and input correct diskette driver name.

4. Follow the instructions on the screen. "SETUP.EXE" program will

retrieve NDIS driver programs from the NDIS driver disk and put

them on the harddisk and directory where LAN Manager resides.

5. Reboot your computer and follow NDIS binding procedure to use

the DC21X4.OS2 NDIS driver programs.

--- Sample PROTOCOL.INI file ---

[PROTOCOL MANAGER]

DRIVERNAME = PROTMAN$

[NETBEUI_XIF]

Drivername = netbeui$

SESSIONS = 40

NCBS = 85

BINDINGS = "DC21X4_NIF"

[DC21X4_NIF]

DRIVERNAME = DC21X4$

SIA_Mode = AUTOSENSE