NDIS Driver Installation Guide for Banyan VINES client

============================================================

Introduction

============

This document describes the procedure to install NDIS 2.01 driver on

"Banyan VINES" DOS client for AEF-380TXD PCI Fast Ethernet Adapter.

This driver conforms to the Network Driver Interface Specification

(NDIS 2.01) defined by Microsoft/3Com.

Installation Procedure

======================

1.  Use the DOS MD command to create a subdirectory called \VINES.

2.  Copy the contents of the VINES LAN S/W (DOS) Install diskette

into the \VINES subdirectory.

3.  Copy the contents of the NDIS subdirectory on the VINES LAN S/W

(DOS) 2 of 2 diskette into the \VINES subdirectory.

4.  Copy the DC21X4.DOS file from \LANSRVR\DOS sub-directory on the

driver diskette to the \VINES subdirectory.

Note : The DC21X4.DOS driver is under \LANSRVR\DOS sub-directory.

5.  Edit the PROTOCOL.INI file as follows:

[PROTOCOL MANAGER]

DRIVERNAME = PROTMAN$

[VINES_XIF]

DRIVERNAME = NDISBAN$

BINDINGS = DC21X4_NIF

[DC21X4_NIF]

DRIVERNAME = DC21X4$

SIA_Mode = AUTOSENSE

6.  Run the PCCONFIG.EXE program located in the \VINES subdirectory.

7.  Select option 1, Network Card Settings.

8.  Select the "NDIS Ethernet" option.

9.  The NDIS workstation screen appears. Enter the IRQ that you

selected for the 10/100 Mbps PCI Fast Ethernet adapter.

10.  Press 'F10' when done, press <ESC> to exit this screen.

11.  Select "2 - Login Environment Settings. ", Press <Enter>.

12.  Select "1 - Select Default Communications Driver. ", Press <Enter>.

13.  Choose "NDIS Ethernet", Press <ESC> to exit this menu.

14.  Edit the CONFIG.SYS file and add the following lines, substituting

A: for C: for a floppy diskette installation.

DEVICE = C:\VINES\PROTMAN.DOS /I:C:\VINES

DEVICE = C:\VINES\DC21X4.DOS

15.  Add the BAN command to the AUTOEXEC.BAT file.  BAN loads the NDISBAN

drivers and reads the PCCONFIG.DB file to invoke changes made during

the PCCONFIG stage.