NDIS Driver Installation Guide for IBM LAN Server

=======================================================

Introduction

============

This document describes the procedure to install NDIS2 driver on IBM

OS/2 LAN Server for AEF-380TXD PCI Fast Ethernet Adapter.

This driver conforms to the Network Driver Interface Specification

(NDIS 2.01) defined by Microsoft/3Com.

Installation Procedure

======================

A. Installing driver procedures for IBM LAN Server 2.0 or 3.0 :

Before you start with the installation  process,  make sure that

OS/2 LAN Server  is properly installed.  Similarly, your adapter

should also be properly installed in your server.

1. Copy DC21X4.OS2 and DC21X4.NIF files from your Driver diskette

(\LANSRVR\OS2) to C:\IBMCOM\MACS.

2. Click the OS/2 LAN Service LAPS icon or manually run LAPS.EXE

from the IBMCOM directory.

3. Select "Installation and configuration".  The IBM logo should

appear on screen.

4. Click <OK>.

5. Select <Advance> and press <Enter>.

6. Select "Install or Configuration this Workstation."

7. Select server type, adding a server or domain controller. The

"install or remove" selections should appear.

8. Select "Configure a component."

10. Click the "Configure" button.

11. Select "Configure workstation."

12. Select "AEF-380TXD PCI Fast Ethernet Adapter" when prompted to

identify you Network Adapter. Then click the <Add> button.

13. Select IBM NETBIOS from the list and click the <Add> button.

14. Select <OK> to confirm all selections.

15. Follow screen instructions to complete the process, selecting

<OK> to accept default selections.

16. Installation is complete now. Reset your PC by turning it off

and on.

B. Installing driver procedures for IBM OS/2 LAN Server 4.0 :

Before you start with the installation  process,  make sure that

OS/2 LAN Server  is properly installed.  Similarly, your adapter

should also be properly installed in your server.

1. In OS/2 window, Click on the "MPTS" icon.

2. The screen will display the IBM Log, press "OK".

3. Select "Install" button.

4. Insert the driver disk, specify the driver path A:\LANSRVR\OS2

and press <Enter>.

5. When complete above step to load DC21X4.OS2 into OS/2 system,

then configure your system.

6. The same step as step 3, select "Configure" button.

7. The screen will display "Configure" dialog box and select "LAN

adapters and protocols" then press "configure" button.

8. The screen will display "LAPS Configuration" dialog box, move

highlight to "AEF-380TXD PCI Fast Ethernet Adapter" item in

"Network Adapter" window, and select "NetBios" protocol. Then,

press "OK".

9. Installation is complete now. Reset your PC by turning it off

and on.