Driver Installation Guide for Windows 95

==============================================

Introduction

============

This document describes the procedure to install NDIS 3.0 driver on

MS Windows 95 for AEF-380TXD PCI Fast Ethernet Adapter.

This driver conforms to the Network Driver Interface Specification

(NDIS 3.0) defined by Microsoft.

Installation procedure

======================

The DC21X4 based adapters are autodetected by Windows 95.

To install the driver follow the menu driven driver's installation.

If You want to install the driver by self, follow steps below :

1. When log into Windows 95, click on the "Start" icon ,then move

the mouse to "Settings" icon and click on the "Control Panel".

2. In the Control Panel window, click on the "Network" icon.

3. If DC21X4 based adapters are autodetected by Windows95, you must

remove the DC21X4 based adapters from Windows95. Do the follow

a. In the Network Configuration dialog box, select the adapter

name, and choose 'Remove' button.

b. Choose 'OK' button.

c. In System Settings Change dialog box, don't restart your PC,

select 'No' button.

4. In the Network Configuration dialog box, choose the "Add.." button.

5. When the Select Network Component Type dialog box appears, select

"Adapter" icon and then press "Add" button.

6. In the Selected Network adapters dialog box, Choose the "Have disk"

button.

7. Insert the driver diskette into drive A or B.

8. When the Install From Disk dialog box appear, specify the driver

path (A:\WIN95 or B:\WIN95) and then choose "OK" button.

9. Select your option

[AEF-380TXD PCI Fast Ethernet Adapter (DC21140) ]

from the dialog box, then select OK.

10. The Windows 95 OS begin to copy driver from diskette and some system

files from Windows 95 installation CD or diskette.

11. Then, you will see adapter list that you had been selected in the

Network Configuration dialog box, select it and then click on the

"Properties" button.

12. In this dialog box, select "Resources ",the screen will come up with

settings. Make sure the values that match you want.

13. Shutdown your system.

14. Turn the power off.

15. Restarting your system you will acquire network service.

NOTE: It is recommended to turn the power off before restart your system.

Parameters

==========

-------------------------------------------------------------------------

AEF-380TXD PCI Fast Ethernet Adapter (DSChip 21140)

-------------------------------------------------------------------------

o AutoSense                   Senses dynamically the media ports

o AutoSense No_Nway           Senses dynamically the media ports

without Nway negotiation

o 100BaseTx                   To connect 100BaseTx Ethernet hubs

o 100BaseTx Full_Duplex       To connect 100BaseTx Full Duplex hubs

o 100BaseT4                   To connect 100BaseT4 Ethernet hubs

o 100BaseFx                   To connect 100BaseFx Ethernet hubs

o 10BaseT                     To connect 10BaseT Ethernet hubs

o 10BaseT Full_Duplex         To connect 10BaseT Full Duplex hubs

BurstLength =   {length}

CSR0<PBL>  1  doubleword

2  doublewords

4  doublewords

8  doublewords

16  doublewords

32  doublewords   (not DC21040 & DC21041)

InterruptMitigation = {Enabled | Disabled} (DC21140,DC21142)

Enable/disable the Interrupt Mitigation

algorithm.

InterruptThreshold = {number_of_interrupt/second} (default: 500)

Maximum number of interrupt/second allowed before

the Receive and Transmit interrupts get masked.

Meaningfull only when InterruptMitigation is enabled.

FrameThreshold = {number_of_frame/second} (default: 400)

Minimum number of processed frame/second allowed

before the Receive and Transmit interrupts get

unmasked.

Meaningfull only when InterruptMitigation is enabled.

MapRegisters =  {4,8,16,32,64}

Number of map registers allocated

by the adapter.

MemoryReadMultiple = { Disabled | Enabled }  (DC21142, DC21143only)

Enable/disable PCI Memory_Read_Multiple command.

MRM should be enabled when the DC21X4 is connected

to the TI1130 PCI/CardBus bridge.

NwayProtocol = { Enabled | Disabled }  (DC21041 and

DC21142 Rev1.0,1.1 only)

Enable/disable the 10BT/10BT_Full Duplex Nway

medium negotiation in Autosense mode.

ReceiveBuffers = {8..64}

Number of buffers allocated by the

driver in the Receive ring.

StoreAndForward =  {Enabled|Disabled} (DC21140 only)

Select the StoreAndForward mode if high

PCI latencies causes 100Mbps Transmit Underruns

CSR6<SF>    1 Enabled

0 Disabled

TransmitThreshold = {value}

Transmit Threshold values for 10Mbps media

CSR6<TTM,TR>  72 bytes

96 bytes

128 bytes

160 bytes

TransmitThreshold100 =  {value} (DC21140 only)

Transmit Threshold values for 100Mbps media.

Meaningless when StoreAndForward is set

CSR6<TTM,TR> 128 bytes

256 bytes

512 bytes

1024 bytes

SnoozeMode =    {Enabled|Disabled} (DC21041 only)

CFDA<SM>    0 Disabled

1 Enabled

UnderrunRetry =  {2,4,6,8}

Maximun number of retransmission attempts of

an underrun packet.

UnderrunThreshold = {10,20,50,100} (21140,21142,21143 only)

Maximun number of underrun occurences before the

controller switches to StoreAndForward mode.