LLI Driver Installation Guide for SCO UNIX

================================================

Introduction

============

This document describes the procedure to install the SCO UNIX LLI

driver for AEF-380TXD PCI Fast Ethernet Adapter.

Software Revision Requirements:

-------------------------------

ODT 2.0 or higher version, or

SCO Unix 3.2.4 or higher version, or

SCO OpenServer 5.0 or higher version

Installation Procedure

======================

1. Copy the file SUL332TD.Z from driver disk to SCO Unix machine.

# doscp -r A:/SCO_UNIX/LLI/SUL332TD.Z  SUL332T.dd.Z    ; driver 0

or

# doscp -r B:/SCO_UNIX/LLI/SUL332TD.Z  SUL332T.dd.Z    ; driver 1

2. To verify, use:

# sum -r SUL332T.dd.Z

3. Uncompress SUL332T.dd.Z to SUL332T.dd, using:

# uncompress SUL332T.dd

4. Copy the file to a diskette. To create a 3.5" floppy in drive 0 or 1,

use:

# dd if=SUL332T.dd of=/dev/rfd0135ds18      ; drive 0 (drive A)

or

# dd if=SUL332T.dd of=/dev/rfd1135ds18      ; drive 1 (drive B)

Mark the diskette:

"AEF-380TXD PCI Fast Ethernet Adapter LLI driver, Rel.03.20"

5. Run custom. From the menu select:

Custom -> Install -> A New Product -> Entire Product

For SCO OpenServer (Everest), run the software manager. From the

menus select :

Software -> Install New -> From Host -> Media Floppy -> Full

6. Put the distribution diskette in the drive and select continue.

7. When prompted for distribution floppy 1, just press enter.

8. When custom has completed, remove the diskette and quit custom.

9. Run netconfig. Add the desired chain. The DC21x4 drivers are called:

d21x0, d21x1, d21x2 and d21x3. Remember that d21x0 must be configured

first, before any other driver is configured.

NOTE: For boards with more than one DECchip 21x4x on them, a driver

must be installed for each active port on the board. Thus,

if a board has two separate ports, two separate chains must

be created in order to connect both ports to networks. This

is similar to installing two boards, each with one port.

Not all ports must be active. If only one port is used,

it must be the first port (as it is found on the bus by

the PCI BIOS).

10. When the chain has been configured, select 'q' from the netconfig menu.

If you don't wish to modify the space.c file relink the kernel at the

prompt.

11. If you wish to change parameters in the space.c file, edit

/etc/conf/pack.d/d21x0/space.c and make the required changes.

The changes will apply to all the installed boards.

NOTE: If your system does not support 32-bit protected mode PCI

BIOS calls, you MUST modify the parameter XX_HW_ACCESSES

to the PCI mechanism appropriate to your system (see space.c

for details).

12. To activate the adapter, you must reboot. When the system boots, you

should see the following message among the other configuration messages:

%d21x0  0x2000-0x2200  5  -  type=DC21041/TP addr=12-34-56-78-9a-bc

or

%d21x0  0x2000-0x2200  5  -  type=DC21140/100Tx addr=12-34-56-78-9a-bc

with the appropriate I/O addresses, IRQ, serial media and IEEE address.