NDIS Dirver Installation Guide for PATHWORKS for DOS client

=================================================================

Introduction

============

This document describes the procedure to install NDIS 2.01 driver on

"PATHWORKS for DOS" client for AEF-380TXD PCI Fast Ethernet Adapter.

This driver conforms to the Network Driver Interface Specification

(NDIS 2.01) defined by Microsoft/3Com.

Installation Procedure

======================

The Adapter are assumed to have been installed in your computer and

physically connect to the network,  and the DEC "Pathworks for DOS"

is assumed to be installed on your hard disk.

1. Insert the "DEC Pathworks Client Setup Disk #1" into your floppy

driver (A or B).

2. Under DOS prompt, type "NETSETUP" <Enter>.

3. Unless you have an LK250 keyboard, Select "NO".

4. Select the DECNETT transport option and press <Enter>.

5. Insert the DNET TRNSPRT disk #1 into floppy driver.

6. Specify the destination driver letter (C or D).

7. Are you creating an initial workstation diskette? Select "NO".

8. Type the Node Name and Address (i.e. Node Number), Press <Enter>.

9. Type the Server Name and Address (i.i. Node Number), Press <Enter>.

10. Select "Ethernet" when prompt " Select the network adapter type".

11. Select "(Other NDIS)" when prompt "Select the network adapter".

12. Enter correct driver and path name for NDIS driver.

For example:

A:\LANSRVR\DOS\DC21X4.DOS

Note : The DC21X4.DOS driver is under \LANSRVR\DOS sub-directory.

13. If they are all accurate, select "WRITE KEY DISK" to start insta-

llation. Follow all prompts till completion.

14. At the configuration screen, press F10 to return DOS prompt.

15. Unsing text editor to edit PROTOCOL.INI at DECNET sub-directory and

CONFIG.SYS under root directory as following.

--- Sample CONFIG.SYS file ---

:

DEVICE=C:\DECNET\PROTMAN.SYS /I:C:\DECNET

DEVICE=C:\DECNET\DC21X4.DOS

:

--- Sample PROTOCOL.INI file ---

:

[IPX4MAC]

DRIVER = IPX$MAC

BINDINGS = DC21X4_NIF

[DATALINK]

DRIVERNAME DLL$MAC

LG_BUFFERS = 16

SM_BUFFERS = 6

OUTSTANDING = 32

HEURISTICS = 0

BINDINGS = DC21X4_NIF

[DC21X4_NIF]

DRIVERNAME = DC21X4$

SIA_Mode = AUTOSENSE