Packet Driver Installation Guide

======================================

Introduction

============

This document describes the procedure to install  Packet Driver  for

AEF-380TXD PCI Fast Ethernet Adapter.

The DC21X4.COM program is complies with the  FTP PC/TCP  Packet Driver

Specifications. This packet driver allow you to use the AEF-380TXD PCI

Fast Ethernet Adapter with most of the DOS version's TCP/IP software.

Installation Procedure

======================

A. DC21X4 syntax:

DC21X4  [option] <PD_INT_NO>

option:

-d    The "-d" option does not start LAN card until this driver is

called by the higher layer programs first.

-n    Your  NOVELL server uses NOVELL ISO-like packets. Your NOVELL

workstation uses  NOVELL 8137 packets. You can use this option

to convert NOVELL 8137 packets into  NOVELL ISO-like packets.

-w    Steve Wallace added a hack to let the packet drivers run under

Windows. This is not to be construed with the proper solution,

which is to write a TCP/IP package for Windows.

This hack is enabled only when you use the '-w' switch.

-u    This option unloads the Packet Driver.

-h    This option displays the on-line help.

-slot=<number>

This is the number of assigned slot, which is between 1 and F.

-turbo This option is to disable PCI post write in Intel PCI chipset

which gives the PCI bus higher priority over the other buses

(EISA or ISA).

-autosense

This option let you change media without restarting the

driver. It's the default setting.

-tp   This option force the driver to select Twisted Pair interface.

-bnc  This option force the driver to select BNC interface.

-tp_full_duplex

This option select Twisted Pair interface running in full

duplex mode.

-tp_no_link_test

This option select Twisted Pair interface. Transmit does not

perform link pulses test.

-line_speed=<number>

This option select Line Speed. Which can be 10 or 100.

PD_INT_NO   This specifies the software interrupt where you will

load the packet driver. It must be in the range 0x60

to 0x80.

Example:       DC21X4 0x60

Note: (1) The PC/TCP kernel, by default, is loaded at interrupt 0x61

(so don't load the packet driver there!)

(2) The default transmission speed is auto-detection.

(3) The placement of 0x prior to the parameters.

B. Run PC/TCP application:

After the Packet Driver is loaded, you can run the PC/TCP kernel

ETHDRV.EXE. The PC is then ready for PC/TCP communications.

1. Refer to PC/TCP manual, Command Reference section, Chapter 2.3.

2. The following example will load the Kernel and allocate 8 packet

buffers.  Increasing the # of packet buffers (from the default of

5) prevents dropping packets due to a buffer shortage.

ETHDRV -p 8 <Enter>

3. At this point, you are ready to run PC/TCP applications

(e.g., 'ftp', 'telnet').

example:  ftp 130.172.24.101

Note:     130.172.24.101  is host address.

Regard to host address , you must know its address.