NDIS Driver Installation Guide for DOS LAN Requester

==========================================================

Introduction

============

This document describes the procedure to install NDIS driver on DOS

LAN Requester for AEF-380TXD PCI Fast Ethernet Adapter.

This driver conforms to the Network Driver Interface Specification

(NDIS 2.01) defined by Microsoft/3Com.

Installation Procedure

======================

A. Installing driver procedures for DOS LAN Requester of LAN Server 2.0

or 3.0 :

1. Insert the IBM LAN Support Program (LSP) Version 1.30 (or 1.31)

Diskette into your floppy drive A: .

2. Type A:\dxmaid  and press <Enter> .

3. The screen will display the IBM logo. Press <Enter> twice until

the "Environment Information" appears on screen.

4. Responds to the screen prompts, specifying the following values

for each field,use the <Space Bar> to toggle between choices:

Configuration for this computer?          Yes

Use existing configuration information?   Yes

Do you have adapter option diskettes?     Yes

Are you configuring for two adapters?     No

Do you need 802.2 interface support?      Yes

Source for LSP                           A:\

Target for new configuration             C:\lsp

Make  sure  that  the values given above are correct, then press

<Enter> to store your choices.

5. Insert the driver diskette in floppy A, and specify your pathname

A:\LANSRVR\DOS, then press ENTER.

6. Follow  screen  instructions  to  complete  the  process. Simply

pressing <Enter> for each screen display will select the default

options. Press <Enter> until you reach the display prompting you

to insert the LSP diskette.

7. Insert the LSP diskette as prompted. Press <Enter> to acknowledge

completion of the action.

8. A screen  showing  the  current  configuration  detected will be

displayed. It should show the Adapter and the protocols already

selected (i.e., "IBM IEEE 802.2" and "IBM OS/2 NETBIOS").

9. Press <F4>. to save the new configuration. This completes the LSP

installation.

10. Save the configuration and reboot your computer.

11. Insert the DOS LAN Requester Install Diskette 1 into your floppy

drive.

12. Type A:\INSTALL    <Enter>

13. The IBM logo will again appear on the screen. Press <Enter> to

move on to the next screen.

14. Follow  screen  instructions,  pressing  <Enter>  to move from

screen-to-screen until you are prompted to indicate the pathname

of the DOS LAN Requester directory (C:\DOSLAN ).

15. Specify the pathname (C:\DOSLAN is the default) for the DOS LAN

Requester directory . Then press <Enter> to continue.

16. The program will ask you for the services that you require.

17. Highlight the appropriate option. Select either "Send messages"

or "Send, View, Edit, and Log message" option. (Normally, second

option "Send, View, Edit and Log message" is selected).

18. Specify the PC's ID and domain ID.

19. Follow screen prompts to complete the installation.

Note : DOS version must be bellow DOS 5.0 when installing DOS LAN

requester into your DOS system.

B. Installing driver procedures for DOS LAN Requester of LAN Server 4.0 :

Before you start with the installation  process,  make sure that

DOS LAN Services is properly installed.  Similarly, your adapter

should also be properly installed in your client PC.

1. Copy DC21X4.DOS from your Driver diskette (A:\LANSRVR\DOS) to DOS

LAN Services subdirectory C:\DLS .

2. Under DOS LAN Requester subdirectory C:\DLS, run INSTALL.

3. The  screen  display  some description. If it's correct, move

highlight to "The listed options are correct" item ,then press

ENTER.

4. Move highlight to "Network Card", then press ENTER.

5. Select "Change driver for network card." item, then press ENTER.

6. Select "Network card not shown in list below..", then press ENTER.

7. Insert the driver diskette in floppy A, and specify the pathname

A:\LANSRVR\DOS, then press ENTER.

8. DC21X4.DOS NDIS driver is now installed on your computer, reboot

your computer.