Driver installation Guide for Novell NetWare Client 32

============================================================

Introduction

============

This document describes the procedure to install 32-bit ODI driver on

"Novell NetWare Client 32"  for AEF-380TXD PCI Fast Ethernet Adapter.

Installation Procedure

======================

A. Install Client 32 for Windows 95 :

NOTE: Please install Novell Netware Client 32 for Windows 95 first.

Method 1: First-time Installation

---------------------------------

If you have previously installed DC21X4.LAN driver of the Netware

Client 32 under your Windows 95 system, skip this section and see

"Follow-up Installation" below.

1. Make sure there is no LAN Card plugged on your machine.

2. Start Windows 95 and get into "Control Panel".

3. Double click "Network" icon, then at the "Configuration" screen

remove all items in list box, after that click "Add" button.

4. On "Select Network Component Type" screen, click "Adapter" which

locates at list box and then click "Add" button

5. On "Select Network Adapters" screen, click "Have Disk" button.

6. On "Install From Disk" screen, input the path where the driver

locates, ex. A:\CLIENT32 or B:\CLIENT32 or

You can click "Browse" button to search for the path where the

driver locates.

7. Another "Select Network Adapters" will be popped up and the

following list is displayed:

[ AEF-380TXD PCI Fast Ethernet Adapter (DC21140) ]

Click the card type which you want to installed and click the

"OK" button.

8. Follow screen instructions until installation is finished.

9. Don't restart your Windows 95 system right now.

10. Get into "Control Panel" again.

11. Double click "Network" icon, then at the "Configuration" screen

remove all items in list box and click "OK" button.

12. Power off and insert AEF-380TXD PCI Fast Ethernet Adapter into

machine.

13. Start Windows 95, then AEF-380TXD PCI Fast Ethernet Adapter will

be found.

14. In "Copying files ..." screen, input the path where the drivers

locates, ex. A:\CLIENT32 or B:\CLIENT32

15. Follow screen instructions until installation is finished.

16. Reboot your system.

Method 2: Follow-up Installation

--------------------------------

If you are re-installing or updating  DC21X4.LAN driver of Novell

Netware Client 32 under Windows 95 system,	follow the procedures

given here. If you have not previously installed DC21X4.LAN driver

of Novell Netware Client 32 under your Windows 95 system,  see the

"First-time Installation" above.

1. Power off and insert AEF-380TXD PCI Fast Ethernet Adapter into

machine.

2. Start Windows 95, then AEF-380TXD PCI Fast Ethernet Adapter will

be found.

3. In "Copying files ..." screen, input the path where the drivers

locates.  ex. A:\CLIENT32 or B:\CLIENT32

4. Follow screen instructions until installation is finished.

5. Reboot your system.

B. Client 32 for DOS/Windows 3.1x :

Method 1: Install Driver During Installing Client 32

----------------------------------------------------

1. Follow the installation setps of Novell Netware Client 32 for

DOS/Windows 3.1x.

2. When 'Insert the NETWARE CLIENT 32 FOR DOS/WINDOWS 3.1x DISK 5 -

LAN DRIVER' diskette ...etc. is prompted insert the desired

diskette and on "Insert The Driver Disk" screen input the path

where the driver locates:  ex. A:\CLIENT32 or B:\CLIENT32

3. Select the desired parameters and continue the procedure.

4. After installation is finished, the following files will be created:

"\NOVELL\DOSCLNT" is installed path of Client32 for DOS/Windows 3.1x.

Example of STARTNET.BAT (for 32-bit LAN driver) ---

SET NWLANGUAGE=ENGLISH

C:\NOVELL\DOSCLNT\NIOS.EXE

LOAD C:\NOVELL\DOSCLNT\LSLC32.NLM

LOAD C:\NOVELL\DOSCLNT\CMSM.NLM

LOAD C:\NOVELL\DOSCLNT\ETHERTSM.NLM

LOAD C:\NOVELL\DOSCLNT\DC21X4.LAN FRAME=ETHERNET_802.2

LOAD C:\NOVELL\DOSCLNT\DC21X4.LAN FRAME=ETHERNET_802.3

LOAD C:\NOVELL\DOSCLNT\DC21X4.LAN FRAME=ETHERNET_II

LOAD C:\NOVELL\DOSCLNT\DC21X4.LAN FRAME=ETHERNET_SNAP

LOAD C:\NOVELL\DOSCLNT\IPX.NLM		<-- Binding driver

LOAD C:\NOVELL\DOSCLNT\CLIENT32.NLM	<-- Finding server

Example of NET.CFG ---

Link Driver DC21X4

Frame Ethernet_802.2

Frame Ethernet_802.3

Frame Ethernet_II

Frame Ethernet_SNAP

NetWare DOS Requester

FIRST NETWORK DRIVE = I

NETWARE PROTOCOL = NDS BIND

NOTE: You can run  the Netware client installation program	for each

workstation type from the CLIENT directory under SYS:PUBLIC or

directly from the CD-ROM.

Method 2: Install Driver after Client 32 is Installed

-----------------------------------------------------

1. Use text editor to edit STARTNET.BAT directly.