Driver Installation Guide for MS LAN Manager v2.x (DOS)

=============================================================

Introduction

============

This document describes the procedure to setup NDIS 2.01 driver on

LAN Manager v2.x DOS Workstation  for AEF-380TXD PCI Fast Ethernet

Adapter.

This driver conforms to the Network Driver Interface Specification

(NDIS 2.01) defined by Microsoft/3Com.

Installation Procedure

======================

A. Suppose you install MS LAN Manager on a DOS workstation for the

first time.

1. Execute the "SETUP.EXE" program that is provided by LAN Manager.

Upon completion of retrieval of the last workstation disk's

contents, you will be asked if you want to import additional

network drivers.  Answer "YES".

2. Insert this diskette into the floppy drive.

3. Follow the instructions on the screen. "SETUP.EXE" program will

retrieve NDIS driver programs from the diskette and put them on

the harddisk and directory where LAN Manager resides.

4. After completion of the setup procedure,  your CONFIG.SYS file

will contain statements for the DC21X4.DOS NDIS driver programs

as sample file.

5. Reboot your computer and follow NDIS binding procedure to use

the DC21X4.DOS NDIS driver programs.

B. If you have installed LAN Manager in your harddisk.

1. Execute the "SETUP.EXE" program.

2. On the screen, select 'Configuration', 'Network Drivers', 'other

Drivers' function and input correct diskette driver name.

3. Insert driver diskette into the floppy drive.

4. Follow the instructions on the screen. "SETUP.EXE" program will

retrieve all NDIS driver programs from the driver diskette and

put them on the directory where LAN Manager resides.

5. After completion of the setup procedure,  your CONFIG.SYS file

will contain statements for the DC21X4.DOS NDIS driver programs.

6. Reboot your computer and follow NDIS binding procedure to use

the DC21X4.DOS NDIS driver programs.

--- Sample CONFIG.SYS file ---

:

DEVICE=C:\LANMAN.DOS\DRIVERS\PROTMAN\PROTMAN.DOS /i:C:\LANMAN.DOS

DEVICE=C:\LANMAN.DOS\DRIVERS\ETHERNET\DC21X4\DC21X4.DOS

:

--- Sample PROTOCOL.INI file ---

[PROTMAN]

DRIVERNAME = PROTMAN$

DYNAMIC = YES

PRIORITY = NETBEUI

[NETBEUI_XIF]

Drivername = netbeui$

SESSIONS = 6

NCBS = 12

BINDINGS = "DC21X4_NIF"

LANABASE = 0

[DC21X4_NIF]

DRIVERNAME = DC21X4$

SIA_Mode = AUTOSENSE