Driver Installation Guide for Windows NT 3.51

==================================================

Introduction

============

This document describes the procedure to install  NDIS 3.0 driver

on Windows NT 3.51 for AEF-380TXD PCI Fast Ethernet Adapter.

This driver conforms to the Network Driver Interface Specification

(NDIS 3.0) defined by Microsoft.

Installation Procedure

======================

1. If Windows NT operating system is being installed, select other

manufacturers from the network adapter card options and follow

steps 3 and later.

2. If Windows NT operating system is already installed, logon as

Administrator and select Netwrok icon from the control panel.

3. In the Network Setting dialog box, choose "Add Adapter" button.

The Add Network Adapter dialog box appears.

4. In the Add Network Adapter dialog box, pull the adapter card list

menu and select "<Other>Requires disk from manufacturer" from the

list, and then press <Enter> button. .

5. Insert the driver diskette in Drive A/B and place the Windows NT

CDROM in the CD-ROM reader.

6. In the next windows, specify the correct drive & path (A:\WINNT40

or B:\WINNT40) when asked to give the source for the driver,  and

then choose the OK button.

Note: The driver is under \WINNT40 sub-directory, because same as

Windows NT 4.0.

7. Select your option

[AEF-380TXD PCI Fast Ethernet Adapter (DC21140) ]

from the Select OEM Option box, then select OK.

8. For PCI options, a Dialog box pops up to select the:

ConnectionType:

----------------------------------------------------------------------

AEF-380TXD PCI Fast Ethernet Adapter (DC21140)

----------------------------------------------------------------------

o AutoSense                     Senses dynamically the media ports

o AutoSense_NoNway              Senses dynamically the media ports

without Nway negotiation

o 100BaseTx                     To connect 100BaseTx Ethernet hubs

o 100BaseTx Full_Duplex         To connect 100BaseTx Full Duplex hubs

o 100BaseT4                     To connect 100BaseT4 Ethernet hubs

o 100BaseFx                     To connect 100BaseFx Ethernet hubs

o 10BaseT                       To connect 10BaseT Ethernet hubs

o 10BaseT Full_Duplex           To connect 10BaseT Full Duplex hubs

9. Finally close the Setup dialog box and exit from the Add Network

adapter dialog box to complete the installation. Follow any

instructions appearing on the screen.

10. Shutdown and reboot the system.