Driver Installation Guide for Windows For Workgroups 3.1

==============================================================

Introduction

============

This document describes the procedures to install NDIS 2.01 driver on

Windows for Workgroups v3.1 for AEF-380TXD PCI Fast Ethernet Adapter.

This driver conforms to the Network Driver Interface Specification

(NDIS 2.01) defined by Microsoft/3Com.

Installation Procedure

======================

A. Install driver when WFW 3.1 doesn't exist.

1. When installing Windows for Workgroups, you are prompted for a

network driver. Select the "Unlisted or Updated Network Driver".

2. Insert the adapter driver disk.

3. When a dialog screen is displayed on the screen to ask the driver

path. Enter "A:\WFW31" or "B:\WFW31" and press return or OK button.

4. After Windows has read the needed files. It will show another dialog

screen. You can select hardware configuration as your adapter settings.

5. Continue the installation per Windows for Workgroups instructions.

B. Install driver when WFW 3.1 already exists.

1. Start your Windows For Workgroups and open main group.

2. Select "CONTROL PANEL" icon in main group.

3. Select "NETWORK" icon in control panel window.

4. Select "ADAPTERS" icon in network window.

5. Select the Add option.

6. Select "Unlisted or Updated Network Adapter" and press OK button.

7. Insert the adapter driver disk.

8. When A dialog screen is displayed on the screen to ask the driver

path. Enter "A:\WFW31" or "B:\WFW31" and press return or OK button.

9. After Windows has read the needed files. It will show another dialog

screen. You can select hardware configuration as your adapter settings.

10. After Windows stores the setting, You can restart your computer to

start the driver.