Driver Installation Guide for Windows For Workgroups 3.11

===============================================================

Introduction

============

This document describes the procedures to install NDIS 3.0 driver on

Windows for Workgroups 3.11 for AEF-380TXD PCI Fast Ethernet Adapter.

This driver conforms to the Network Driver Interface Specification

(NDIS 3.0) defined by Microsoft.

Installation Procedure

======================

1. Turn your computer off, then Install your Ethernet Adapter to your

computer.

2. After the adapter installation, turn your computer on.

3. Start your Windows for Workgroups 3.11.

4. After Windows Start OK, Select "Network" icon and then Select

"Network  Setup" icon.

5. Click the "Drivers..." button.

6. If you are replacing a previous LAN adapter select the Remove button

to delete that adapter

7. Click the "Add Adapter" button.

8. Select "Unlisted or Updated Network Adapter" item bar on the Adapter

list.

9. Insert the driver diskette into drive A: or B: and enter A:\WFW311

or B:\WFW311.

10. Select the adapter that matches your hardware from the Network

Adapters list

[AEF-380TXD PCI Fast Ethernet Adapter (DC21140)]

,then Select "OK" bar.

11. For PCI adapters, the driver's [Setup..] option allows the user to

change the following driver's properties:

* BurstLength:    Specify the length of the DMA data transfer within

a PCI burst (1dword = 4 bytes)

1_dword

2_dwords

4_dwords

8_dwords

16_dwords  (default)

32_dwords  (DC21140 only)

* Bus Number:     Specify the PCI Bus Number the adapter is plugged in.

Should be modified only for adapter plugged over a

PCI to PCI bridge.

0  (default)

1

2

* ConnectionType:   Specify the selected Media port

------------------------------------------------------------------------

AEF-380TXD PCI Fast Ethernet Adapter (DECchip 21140)

-------------------------------------------------------------------------

o AutoSense                  Senses dynamically the media ports

o 100BaseTx                  To connect 100BaseTx Ethernet hubs

o 100BaseTx Full_Duplex      To connect 100BaseTx Full Duplex hubs

o 100BaseT4                  To connect 100BaseT4 Ethernet hubs

o 100BaseFx                  To connect 100BaseFx Ethernet hubs

o 10BaseT                    To connect 10BaseT Ethernet hubs

o 10BaseT Full_Duplex        To connect 10BaseT Full Duplex hubs

* PciMechanism:   Specify the mechanism access to the PCI

configuration space.

o AutoDetect                  Detect the system's PCI mechanism

automatically

o Mechanism 1                 Force PCI mechanism #1 accesses to the

PCI configuration space

o Mechanism 2                 Force PCI mechanism #2 accesses to the

PCI configuration space

* ReceiveBuffers:  Specify the size of the receive buffer's ring

8 .. 64  (default 16)

* MapRegisters = REG_DWORD {4,8,16}   (default: 8)

Number of map registers allocated

by the adapter.

* SnoozeMode = REG_DWORD { 0 | 1 }  (DC21041 only)

CFDA<SM>    0 Disabled (default)

1 Enabled

* StoreAndForward = REG_DWORD { 0 | 1}  (DC21140 only)

Select the StoreAndForward mode if a low PCI

bandwith causes Transmit Underruns

CSR6<SF>    1 StoreAndForward mode

* TransmitThreshold = REG_DWORD { 72 | 96  | 128 | 160 |

256 | 512 | 1024        }

CSR6<TTM,TR>  72 bytes

96 bytes (DC2104X's default)

128 bytes

160 bytes (DC21140's default)

256 bytes (DC21140 only)

512 bytes (DC21140 only)

1024 bytes (DC21140 only)

12. Select "Close" bar and then Select "OK" bar to complete installation.

13. Windows prompts you to insert several WFW system diskettes.

14. When message "Windows Setup has modified your SYSTEM.INI and

PROTOCOL.INI...    ....." appears, select "OK" bar.

15. Select "Restart Computer" bar.

16. Now, the Windows for Workgroup 3.11 is ready for your Ethernet

Adapter.