Run time library of C language for S-LINK multi serial card.

This disk content as follows:

SIODEF.H        Head file for every library

ASCII.H         Head file for ASCII code definitions

SIOCTL.H        Head file for serial I/O controls

U8250.H         Head file for U8250.LIB

MODEM.H         Head file MODEM.LIB

XMOD.H          Head file for XMODEM.LIB

CRC.H           Head file for CRC.LIB

SIO.LIB         Level-3 library

BUOS.LIB        Level-2 library

U8250.LIB       Level-1 library

MODEM.LIB       Library for modem operating

XMODEM.LIB      Library for XMODEM protocol file transfers

CRC.LIB         Library for CRC calculations used in XMODEM

file transfers

IBMPC.OBJ       Level-0 file

YOUR.OBJ        Console I/O

The library hierarchy

SIO.LIB: the level-3 library

The top of library hierarchy resides the level-3 functions. These

functions are generally all that are required for writing programs,

although it is sometimes necessary to call level-2 functions directly

as well.  The level-3 library will be refered to by the name SIO.LIB.

BUOS.LIB: the level-2 library

BUOS.LIB contains functions such as s_putc -- functions whose respon-

sibility is to provide an interface between the hardware and the

level-3 functions.

"UARTS".LIB: the level-1 library

Since the UART used in any application dominates the character of

serial I/O, we require a level-1 library for each UART  for which we

intend to write code.  Here reside the level-1 primitives that are

called by the level-2 functions in BUOS.LIB.  In other words, the

functions in these UART libraries are responsible for resolving

hardware differences for the level-2 functions.  The level-1 library

will be refered to by the name U8250.LIB for the IBM PC AT.

"COMPUTER".C: The level-0 files

This level-0 file contains computer-specific modules that is the

functions at every other level must refer to it. The level-0 file

will be refered by the name IBMPC.C.

The \SOURCE directory:

All the libraries are compiled by small memory mode.  If you need to

build your program in other memory mode, you have to recompile the

libraries using the library source file contained in this directory.

Every module in source file has same following profile you can get the

information on how to call it:

/*

FUNCTION NAME:  s_puts

LEVEL:          3

LIBRARY:        SIO.LIB

DESCRIPTION:    Writes a NUL-terminated string to the UART pointed

to in the sio structure.

OTHER MODULES:  s_putc

RETURNS:        void

*/

void s_puts(str)

char *str;

{

for (; *str != NUL; str++)

s_putc(*str);

}

Reduce the program size:

For the convenience, we make every module together in one source file.

For example, the all SIO.LIB module source files are in SIO.C source

file. You can compile the modules separately, or cancel unnecessary

modles to reduce your program size.