When writing program using Multi Serial car library , please link

your object code with MULTI.LIB.

The library functions provided is list below :

int  com_initial(int COM_PORT,int BAUD_RATE,int PARITY,int STOP_BIT,

int DATA_LEN,int IRQ_NO);

Descrption : Initialize Serial Port and set Interrupt Service Routine ready

Uasge :

COM_PORT    RS 232 port to be used

BAUD_RATE   Baud Rate Setting (1200,2400,4800 or 9600)

PARITY      Parity Check Type (0--None,1--Odd,2--Even)

STOP_BIT    Stopt Bit Setting (0 or 1)

DATA_LEN    Data Word Length  (7 bits/dataword or 8 bits/dataword)

IRQ_NO      Interrupt Vector to be used (according to your setting

on Multi-Serial card

Return      -1 if any error occurs

int  HangUp();

Description : Hang Up telephone link (if connect remote site using telephone

line)

Usage :

No argument needed.

Return       1 if hang up successfully;

int  send(char  *request,int  len);

Description : Send data string to Serial Port

Usage :

request    data buffer (string) to be send

len        length of data buffer *request to be send

Return     count of data bytes send;

int  rcve(char  *rcve_buf);

Description : Receive data string to Serial Port

Usage :

rcve_buf   data buffer (string) received

Return     length of data received;

void  com_send(char  data);

Description : Send one char byte data to Serial Port

Usage :

data        char byte to be send

Return      None;

void  sleep(int  nop_time);

Usage :

nop_time    set PC to do nothing for nop_time

Return      None;

int  ReadPortAddr(char *AddrFile)

Descritpion : Read Multi Serial card Port setting

Usage :

AddrFile    Multi Serial card port setting file name.

The format of your address config file must as below :

3f8   2b8

2f8   2b9

4f8   3b8

5f8   4b8

^     ^

|     |

|     +---- Interrupt Vector according

|           to your setting on Multi Serial Card

|

+---------- Serial port address

Return      1 if open successfully; -1 if error occurs;

void  clrscr(int  top,int  bottom);

Description : Clear screen

Usage :

top          left-up corner corordinate of screen to be clear

bottom       right-down corner corordinate of screen to be clear

Return       None;

void  cursor_pos(int x,int  y);

Description : Move cursor to position (x,y)

Usage :

x            Column number of screen (0-79)

y            Row number of screen    (0-24)

Return       None;