# Momo-Notebook USB Link User’s Manual

Introduction

Congratulations on your purchase of the Momo-Notebook USB Link . This unique link allows you to easily connect two computers with USB port and instantly transfer files at great speed and ease.

Package Checklist

The Momo-Notebook USB Link  product package includes the following items:

- Momo-Notebook USB Link

- Driver and Software Installation Diskette

- This user’s manual

Overview

The Momo-Notebook USB Link  is your fast solution to peer-to-peer file transfer between two computers via the USB port. The Momo-Notebook USB Link provides true plug-n-play capability while transferring files at much greater speed than traditional file transfer products via serial or parallel port. You get around 4 to 8 Mbps of smooth file transfer rate. The Momo-Notebook USB Link application program, a Windows Program Manager-like user interface, provides a very easy to use environment that makes file transfer as easy as drag-and-drop. You simply plug in the Momo-Notebook USB Link and do a one-time installation of the program.

Features

The Momo-Notebook USB Link  provides the following features:

- Single chip (ASIC) USB host to host communication

- High-speed data transfer rate from 4Mbps to 8Mbps

- Single cable solution for data communication

- Windows 98 and 2000 Support

- Easy drag-n-drop file transfer

- Includes print function for remote files

- Includes power management and remote wake-up functions

- Compliant with USB specification 1.1

System Requirements

Before you use the Momo-Notebook USB Link , make sure your computer is an IBM PC-compatible with the following minimum system requirements:

- Intel-compatible 486DX-66 MHz CPU or higher

- One standard USB port (4-pin) and has the symbol

- Either the following operating systems:

- Windows 98

- Windows 2000

Driver Installation

Follow the steps below on installing the Momo-Notebook USB Link for the first time:

- Power on both computers where you will connect the Momo-Notebook USB Link and make sure that the USB port is enabled and working properly.

- Plug in the USB cable into the USB port and Windows will detect an unknown device and run the Add New Hardware Wizard to assist you in setting up the new device.

- Insert the Momo-Notebook USB Link driver diskette into the floppy drive and click Next to continue.

- For WINDOWS 98 users:

- Select Search for the best driver for your device and click Next.

- Select Specify a location and click Browse. Change the folder of your floppy drive (ie:A:\ WIN98) and click OK.

- Double check the directory that Windows prompts. Click Next.

- Windows will detect the driver (usbbc.inf) and shows the USB Bridge Cable (Windows 98). Click NEXT to continue until installation is complete.

- Click Finish while installation is complete.

- For Windows 2000 users:

- Select Search for the best driver for your device and click Next

b.      Select Specify a location and click Browse. Change the folder of your floppy drive (ie:A:\ WIN2000) and click OK.

- Double check the directory that Windows prompts. Click Next.

- Windows will detect the driver (usbbc20.inf) and shows the USB Bridge Cable (Windows

2000). Click NEXT to continue until installation is complete.

- Click Finish while installation is complete.

USB-Link Program Installation

After installing the device driver for the Momo-Notebook USB Link ,  you need to install the USB-Link application program for transferring files:

- Insert the Momo-Notebook USB Link driver diskette into the floppy disk drive and run the SETUP program.

- The USB-Link Setup program will start to begin program installation.

- Follow the instructions on every dialog box to finish the installation.

- After installation is complete, a shortcut of the USB-Link program will be placed on your Windows desktop.

Repeat the same procedure on the other computer.

Running the USB-Link Program

Follow the steps below on how to run the USB-Link program:

- Plug in both ends of the Momo-Notebook USB Link to the two computers.

- Double-click on the USB-Link shortcut icon on your desktop screen. Do the same for the other computer.

- The USB-Link File Manager will appear on your screen and will detect both the local and remote machine. This is much similar with Windows Explorer file manager where you can cut, copy, paste, or drag files and folders.

- You can also print text or document files from the remote machine to your local printer by clicking on the file and run File-Print command.

- There are two LED-like indicators found on the bottom-right corner of the USB-Link File Manager. These indicators show the connection status of the remote and local machine. The left LED is for the local machine, while the right LED is for the remote machine. A green LED indicates that the connection is good while a red LED indicates that connection is not found. Both LEDs need to be in green color for a proper link and operation. If one LED is red, either the USB cable is not properly connected, or the software is not properly installed, or the PC is in suspend mode.

- Click on the Help Topics icon (rightmost icon) for more information on how to use the program.

Power management and Remote Wake-up Function

The Momo-Notebook USB Link supports power management of USB. When either of the two connected PC gets into suspend, the Momo-Notebook USB Link will draw less than 500 A of current from it. This is essential for the notebook or other mobile PC.

The USB-Link program also includes the remote wakeup function. Whenever the remote machine goes into suspend mode, you just click on any file or folder inside the remote machine directory and the remote machine will resume to normal operation mode.

However, you can disable this function to avoid the annoyance of suspend-wakeup sequence during operating the USB-Link application. Please click the menu bar’s Option, then click Setting, and disable the Allow System to Suspend option.

Program Uninstallation

You can uninstall the USB-Link application program and driver by selecting the Uninstall USB-Link item from the USB-Link program group and follow the on-screen instructions to complete the process.

Disclaimer

Information in this document is subject to change without notice. The manufacturer does not make any representations or warranties (implied or otherwise) regarding the accuracy and completeness of this document and shall in no event be liable for any loss of profit or any other commercial damage, including but not limited to special, incidental, consequential, or other damages.

No part of this document may be reproduced or transmitted in any form by any means, electronic or mechanical, including photocopying, recording or information recording and retrieval systems without the express written permission of the manufacturer.

All brand names and product names used in this document are trademarks, or registered trademarks of their respective holders.

FCC Statement

This device generates and uses radio frequency and may cause interference to radio and television reception if not installed and used properly. This has been tested and found to comply with the limits of a Class B computing device in accordance with the specifications in Part 15 of FCC Rules. These specifications are designed to provide reasonable protection against such interference in a residential installation. However, there is no guarantee that interference will not occur in a particular installation. If this device does cause harmful interference to radio or television reception, which can be determined by plugging the device in and out, the user can to try to correct the interference by one or more of the following measures:

- Reorient or relocate the receiving antenna.

- Increase the separation between the device and receiver.

- Connect the computer into an outlet on a circuit different from that to which the receiver is connected.

- Consult the dealer or an experienced radio/TV technician for help.